CP110 System disks:

KESH5100.SYS :OS

KESH5400.SYS :data

KESH5500.SYS :data

KESH_600.SYS :data

KESH_700.SYS :data

KESS31F0.SYS :TG OS

KESS51B0.SYS :TG data

Load Procedure for CP110 system disks.

Step 1)

With the power turned off, insert disk #1.

Press and hold the Guitar, Bass, and Synth sound category buttons, then turn on the power. The display will show “Load System File” and loading will begin with a circle bar graph showing the progress. When completed the display will read, “load end”. Turn off the power when loading is complete.

Step 2)

With the power turned off, insert disk #2.

Press and hold the Piano, Organ, and Drawbar sound category buttons, then turn on the power. The loading process will be the same as for disk #1.

Step 3)

Follow the instructions in step 2 (same as disk #2) for all other disks (disk #3-7).

Step 4)

After loading all seven disks perform a factory reset by turning on the power while holding the RESET button located in the Recorder area of the panel.

If you have any problems or questions regarding this O/S update please call Troy or Alan at 800-421-2177 (x337 or 335, respectively).