MP9500 version changes/new features by O/S version

Ver V1.01 g32

First production

# Ver V1.02 g33

1. In Multi mode sounds/program change settings for each zone can now be checked without entering the edit mode.

When you are in Multi mode you can check what sound/program change is assigned to any zone by pressing and holding the zone select switch.

2. Program change receive has been simplified for both Multi and Single mode.

# Single mode

PGM # 1-64 on the TX/RX channel selects the appropriate sound in Single mode.

PGM # 1-64 on any other channel is ignored.

PGM # 65-128 on the system channel switches the MP9500 to Multi mode and selects the appropriate Multi Patch.

PGM # 65-128 on any other channel (including the TX/RX channel if different from system) is ignored.

# Multi Mode

PGM # 65-128 on the system channel selects the appropriate Multi Patch.

PGM # 65-128 on any other channel is ignored.

PGM # 1-64 on the system channel switches the MP9500 into Single mode and selects the appropriate Single sound.

PGM # 1-64 on any other channel, the MP9500 remains in Multi mode and selects the appropriate Single patch for that channel

3. The Factory default settings for Channel Mute/Play in the System mode have been changed to the following:

Channel 1-4 Play.

Channel 5-16 Mute.

5. The spelling of the word "memorizing" has been corrected.

# Ver 1.03 g33a

1. User touch curve problem is fixed.

2. The PGM# display when the zone select button is pressed has been corrected from 0-127 to 1-128.

# Ver V1.1 g39

1. The effect and volume timing when a new patch is selected while notes are still being held has been improved.

2. A new Store parameter, INITIAL CURRENT has been added.

INITIAL CURRENT initializes the edit buffer ONLY. This creates initialized settings in the edit buffer for programming a new patch. The currently selected patch however is NOT initialized so any edits must be stored before they are permanently saved.

3. RESET CURRENT function has been modified.

Previously RESET CURRENT only reset the edit buffer to the original factory settings for the selected patch. Now BOTH the edit buffer and currently selected patch will be reset to the original factory settings by RESET CURRENT.

4. The voice allocation has been improved so that a higher priority is assigned to a note whose key is On when the damper pedal is pressed.